New and more numerous applications continue to increase bandwidth demands in network backbones. Increasingly, high performance servers are the focal points in new network architechures. With no forseeable end to the dramatic growth in network traffic, seasoned IT managers are challenged on several fronts:

- Lots of Hype Over New Networking Technologies

- Need for Long-Term Solution, Not Band-Aid

- Myriad ATM, Switch, Backbone, & Desktop Vendors With Point  Solution Mix

- How To Integrate ATM With Existing LAN Investment...and Manage It All?

Bandwidth demands are increasing as new and more numerous applications come on-line.  While multi-media is touted as a prime driver of fat-pipe solutions, more ubiquitous applications such as e-mail with large-memory attachments  require large amounts of bandwidth, particularly when such items are broadcast through an organization’s network.

Many organizations have moved towards a more centralized server architecture, which places huge demands on the network backbone where the servers reside.

In addition, many desktops are being provided with an internet connection, further burdening the network with graphic-rich traffic.  As desktop computing power continues to increase, there appears to be no relief in site.  Therefore, IS managers must look towards strategic ways of solving the network bottlenecks that exist today while at the same time providing allowances for even more bandwidth demands as the network evolves.

## Practical ATM Solution Strategies

The networking industry has been full of hype regarding new high speed, high bandwidth network technologies such as ATM.  The fact of the matter is that the ATM market has been relatively small until now, because no single vendor has been able to adequately address the real needs of network managers.

Network managers are looking for:

- Long-term solutions, not short-term, point product band-aids

- A relatively complete solution, including ATM backbone, ATM/LAN access, and desktop ATM ingredients

- Assurance that new technologies such as ATM can be safely integrated into existing shared media/switched Ethernet and Token Ring environments

- Assurance that the integrated ATM/LAN network can be managed from a unified application on an open-systems platform

Madge meets all of these criteria. Madge is the only vendor providing complete switched solutions for companies looking to integrate ATM with today’s LANs. The Madge suite of products for the ATM marketplace includes:

- Backbone ATM 155 - Collage 740

- Ethernet-to-ATM - Collage 530, 280, LSA

- Token Ring-to-ATM - Collage 540

- Workgroup ATM 25 - Collage 280

- Desktop Adapters - ATM25, & ATM 155

- Complete System Management

This figure below defines Madge’s integration of ATM and LAN solutions, providing end-to-end switching from Ethernet to ATM, Token Ring to ATM, and ATM desktop to ATM backbone.

This is the “roadmap” which was introduced and presented in the fall of 1995.  Madge is now delivering on the product promises it made in 1995.

# Backbone ATM 155 - Collage 740

The Collage 740 is a true “second-generation” ATM  backbone switch, having been built around more mature standards than previous switches.  The result is a product that conforms to existing ATM Forum standards, but also has the capacity to support evolving standards and future services and demands.

- Investment Protection Through InterOperability

- UNI 3.0, 3.1

- LAN Emulation 1.0

- Top Performance Where Most Needed: Backbone

- 2.4G switch fabric, Non blocking.

- 4-16 ports 155 UTP and fiber

- Support for all traffic classes including ABR

- Hot swap, redundant PSU, distributed LANE server for high reliability

The 740 supports both UNI 3.0 and 3.1 standards, allowing it to fit into either environment.  The product supports LAN Emulation version 1.0 now, and will be upgradeable via software download to LANE 2.0 when the standard is approved.

Not being first to market with an ATM backbone product, Madge has taken advantage of the opportunity to learn from the experiences of other vendors and early adopters.  The Collage 740 is designed for optimum performance under peak load conditions with a completely non-blocking 2.4 Gigabit switch architecture.  This means no lost packets, no missing data.

The switch also supports all four classes of ATM traffic:

- CBR (Constant Bit Rate)

- VBR (Variable Bit Rate)

- UBR (Unspecified Bit Rate)

- ABR/ER (Available Bit Rate)

ABR/ER allows the switch to effectively manage the bandwidth of streams of traffic which contain a mix of delay-sensitive and non-delay sensitive information, such as video and data.  Many switches on the market today do not support ABR/ER and do not have the capability to do so.  ABR/ER support will require a special hardware module and software upgrade, available in 2H96.

The 740 supports connections over category 5 UTP (maximum distance of 100 meters), multi-mode fiber (2 km), and single-mode fiber (15 km).  Each module supports 4 network connections.  From one to four modules can be inserted in the chassis.  Modules can be hot-swapped.  The switch supports primary as well as secondary (back-up) LAN emulation servers.

## Future Enhancements

The Collage 740 provides excellent investment protection for Madge customers, because the 740 is designed for existing as well as future requirements.  The switch is scaleable and stackable.  The following enhancements will be available in the latter part of 1996:

- 3 Gbps switch fabric upgrade

- 622 Mbps ATM inter-switch campus backbone

- ABR/ER flow control upgrade

- Leased-line WAN interfaces

- Enhanced LAN emulation services

- PNNI upgrade

A 3 Gbps switch fabric upgrade will support the demands of a 622 Mbps/OC-12 module.  The module is intended for use in connecting Collage 740 switches together to create a backbone switch solution with greater port density.

The 740 will support ABR/ER flow control and bandwidth management via an available hardware module and a software upgrade.

The 740 will be upgradeable to support a T1/E1 and T3/E3 wide area network interface.

# Ethernet to ATM Backbone
LANswitch LSA Module

The LSA Module protects the LANswitch investment by offering a 155 Mbps ATM connection to ATM backbone switch or another LANswitch.  Fully compliant with the ATM Forum LANE 1.0 and UNI 3.0/3.1, the LSA Module provides the most extensive ATM features, including advanced congestion control and traffic prioritization.  These two features allow for high through-put and support of time sensitive traffic.

- High speed connectivity from MultiNet hub 1.28Gbps Cellenium bus to 155Mbps ATM

- Full support for award winning LANswitch features

- Congestion control

- Traffic prioritization

- Hierarchical VLANs

- SMON

Similar to all Collage products, the LSA Module provides VLAN capability - up to 256 VLANs can be supported.  The module and all the ATM features can be monitored by the highly regarded SMON switched monitoring system developed  by Madge Networks.

Finally, using Madge’s SNMP-based MultiMan network management system, the LSA Module can be managed in the same way as other LANswitch modules.  The advantage is an integrated management for shared, switched and ATM LANs.  In other words, a single management platform for Ethernet, Switched Ethernet and ATM LAN networks.

# Ethernet-to-ATM Access
Collage 530

The Collage 530 is a high performance Ethernet to ATM access switch.  It provides the best migration path from existing Ethernet network to high speed ATM switching. Key features of the Collage 530 include:

- High performance 16 port Ethernet to ATM Access Switch

- Optimized for Ethernet

- Congestion control

- Traffic prioritization

- Hierarchical VLANs

- Port monitoring

- Integrated Management for Shared, Switched and ATM LANs

With built - in Ethernet segment switching, all local Ethernet traffic can be switched among the 16 ports.  Traffic targeted for the backbone ATM network is transported via the high speed 155 Mbps ATM interface using ATM Forum compliant LANE 1.0 and UNI 3.0/3.1 signalling.  Another connection option links two Collage 530 units back to back to create a 155 Mbps ATM backbone.  Designed for maximum interoperability, the Collage 530 also supports Q.2931 signalling.

A total of of 8,192 MAC addresses are supported for maximum end station connections.  Special advanced congestion control features eliminate packet loss and ensure that each packet sent is delivered successfully.  For delay-sensitive traffic such as voice and video, the Collage 530’s traffic prioritization feature guarantees timely delivery of isochronous traffic.

Providing high speed switching without sacrificing broadcast control, the Collage 530 allows up to 16 VLANs to be established.  Virtual LAN function conforms to ATM Forum LANE 1.0 by mapping VLANs to ELANs.  Another unique feature implemented on the Collage 530 is the port monitoring capability.  Port 16 can be configured to mirror traffic from any other port.  This allows an operator non-intrusive monitoring and analysis on live traffic.

To top off the rich standardized features, Collage 530 is managed using Madge’s SNMP-based MultiMan network management system, supported on the most commonly used enterprise management platforms.  The advantage is an integrated management for shared, switched and ATM LANs.  In other words, a single management platform for Ethernet, Switched Ethernet and ATM LAN networks.

# Token Ring-to-ATM Access
Collage 540

The Collage 540 Token Ring to ATM access switch integrates existing 4 & 16 Mbps Token Ring networks with high capacity 155 Mbps ATM backbones using ATM Forum-compliant LAN Emulation (LANE).  The following highlights of the Collage 540 set it apart as THE Token Ring to ATM access switch:

- High Performance 10-port Token Ring to ATM Access

- Optimized for linking multiple Token Rings to ATM backbone/switch

- Redundant 155 ATM Uplinks

- Industry-First Token Ring LAN Emulation

Madge is the first company to provide Token Ring LANE.  Obviously, this will be a key enabler for Token Ring customers wishing to implement ATM in their networks.  LANE allows them to use ATM as a high speed transport mechanism, and does not require any modification of the Token Ring network, nor the addition of any Token Ring-to-ATM protocol conversion device.  Essentially, Token Ring LANE makes ATM plug-and-play in the Token Ring environment.

The 540 supports up to 10 Token Ring ports at 4 or 16 Mbps.  The unit can support redundant 155 Mbps ATM connections over optical fiber.  Support is available for multi-mode (maximum distance 2 km) or single mode (10 km) fiber.

The 540 is optimized for Token Ring to ATM connection (and vice versa).  It is not designed to compete as a frame switch with the Ringswitch.  The 540 supports source route environments now, with plans to provide transparent support underway.

# Madge ATM Management Focus

Madge’s goals and direction with respect to network management are strategically critical for of IS managers. In broad terms, Madge’s network management strategy for ATM includes:

- Reducing the complexity of network management with new applications

- Providing smooth  integration of ATM and Switched/Shared LAN management

- Focus:

- Integrated switching and ATM solution for complete End-to-End management

- ATM Monitoring

- Physical/Logical Topology

IS managers are justifiably concerned about the challenges that ATM management presents. Since the new medium is connection-oriented, it can be difficult to monitor globally, and has the capacity to support massive flows of traffic in the critical backbone segment of the network.

IS managers are also concerned about how to safely integrate new ATM pieces into their existing shared media/switched networks, and want to be able to have everything manageable under one system.

IS managers need systems that can take extensive amounts of sophisticated data, analyze that data, then provide meaningful information in a simplified, easy-to-comprehend format. In addition, ATM network management must address not just physical topologies, but logical topologies as well, as virtual networks  are created and modified.

## Madge’s ATM Management Today

- ATM Backbone Switch Management (Collage 740, LSA)

- Basic Traffic Monitoring and Configuration Control

- Device Management

- Management of ports, PVCs, SVCs, interswitch connections

- ATM Access Switch Management (Collage 540/530)

- ELAN / VLAN logical and physical topology

- Focus on management of end-to-end sessions

Traffic monitoring and configuration control are performed through the highly acclaimed Madge TrueView Collage 740 Manager.  It is supported on the widely accepted HP OpenView and IBM NetView management platforms.

In order to have prime time backbone ATM deployment, per port traffic statistics are required to properly manage the network.  This capability is built into the hardware of the Collage 740 with cell flow statistics collected at real time rates.  All this is on top of the standard port level error and traffic flow statistics which are also present in the switch hardware.

Standards-based SNMP is utilized to manage the Collage 540.  As part of the TrueView network management system, ELAN and VLAN logical and physical topology can be configured in an efficient and intuitive manner.

In addition to the feature rich, standards-based network management system, the Collage 530 allows mirroring of port traffic on port 16.  This unique feature provides an exceptional ability to monitor and manipulate live traffic without interrupting the traffic flow.

## SMON For ATM (AMON)

AMON, ATM Monitor, is an emerging network management standard for ATM which is being drafted by the ATM Forum.  Madge is committed to providing a standards-based management solution for its ATM products.

The figures below show examples of the look-and-feel of Madge’s AMON solution, available first for the Collage 740.

ATM Host Matrix

ATM Switch Statistics

ATM Port, Host and TopN Statistics

Madge’s goal is to provide the first version of an integrated LAN/ATM network management solution on an open-systems platform by 3Q96.

## Desktop ATM Solutions - ATM 155 and ATM 25 to Desktop

Completing the picture of the  end-to-end ATM switched network are the ATM 155 and ATM 25 desktop solutions.

These include the Collage 280, a workgroup switch that can automatically support port speeds of either 10 Mbps Ethernet or 25 Mbps ATM

Madge’s desktop ATM solutions also include 155 Mbps and 25 Mbps ATM adapter cards, as well as LAN emulation software for Ethernet and Token Ring.