# Congestion Management in the LANswitch

## Introduction : Lost Packets affect Available Bandwidth

The Ethernet protocol provides no acknowledge mechanism for confirming data transmissions. Any packet that is transmitted with no collision detected is assumed to have reached its destination successfully. If for any reason, a packet gets lost on the way, a higher network layer must initiate recovery measures.

Rather than acknowledge every single packet, most modern transport protocols allow groups of packets to be acknowledged to reduce the processing overhead. However, it also means that when one packet in a group goes astray, the higher level protocols must process and re-transmit the whole group. When this happens bandwidth is utilized for first-time transmission as well as a second transmission and perhaps even a third. This inefficiency significantly reduces effective bandwidth.

Most switches lose packets when the traffic load increases. As lost packets are re-sent (with upper-layer involvement). more packets are added to the switching queue. As these packets flood the network, port buffers begin to overflow causing further packet loss. Eventually, packet congestion “snowballs” to a debilitating level that ruins network performance.

Only LANswitch controls port buffer loads and guarantees delivery of each and every packet. Even on the busiest ports, no-packet-loss is ensured by a unique congestion management mechanism built into every LANswitch port. LANswitch guarantees that when a packet leaves a station, it will not be lost on the way to its destination. No lost packets means no extra retransmissions and no “snowballing” congestion that could cause the network to halt, or worse, to crash.

The congestion management mechanism essentially uses the end stations own buffers as extensions of the LANswitch buffers. This elegant yet simple approach works with the standard CSMA/CD access method, making congestion control extremely efficient and reliable, resulting in zero packet loss. The overall performance of your network is improved by ensuring that the network is utilized for transmitting first-time packets only, and doesn’t waste time and bandwidth with multiple retransmissions.

## Is There a Traffic Jam at the LANswitch?

Lannet's congestion control mechanism causes the transmitting station to cease transmission when its switch input port buffer runs out of space. The NIC slightly reduces the station’s transmission rate for a brief moment which gives the input port time to regain buffer space. Implementation of this back pressure mechanism results in no lost packets and hence eliminates the "snowballing effect" of packet retransmission discussed above.

When a switched network is configured with one station per switch port, back pressure is unquestionably beneficial and desirable because it halts only the precise station that is causing the transmission overload. However, in configurations where several stations share a switch port, some people contend that back pressure has a detrimental effect because it eliminates all conversation on that port’s segment, effectively blocking local peer to peer communication as well as the current switched conversation.

The response to this claim requires better understanding of the Ethernet environment and the LANswitch back pressure mechanism. First, let’s look at the LANswitch and closely examine the process that is likely to tell stations on the segment to momentarily suspend transmissions.

## Transparent Traffic Control Mechanisms

Assume that a packet is sent from station A on switch port X to station B on 
switch port Y. Also assume that station B (server) is simultaneously accessed by a number of stations (clients) whose combined access demands a higher traffic rate than the server network pipe can provide (due to physical and logical limitations). When this happens, the buffer at the destination port Y will try to compensate for the overflow by storing packets until their turn comes to be forwarded. If the load on the server port never decreases and packets continue to be sent at a higher aggregate input rate than the exit rate, the server port’s buffer will eventually reach its threshold and no more packets will be accepted. Once the buffer’s threshold has been exceeded, the source ports are immediately notified to hold their fire and stop shooting packets at the server port.

Meanwhile, station A is unaware of the server port’s predicament and keeps transmitting through port X. Port X, given the hold-your-fire directive, has no choice but to store all packets while it waits for the server port to free some buffer space. During that brief moment that Port X is storing Station A’s packets, the server port quickly empties its buffer. Since no packets are coming into the server (X stopped transmitting), the “buffered” packets are able to exit at the maximum transmission rate of the server pipe (e.g., 100 Mbps). As soon as the server port empties some of its buffer, source ports (such as “X”) can start sending again, relieving their buffers as well. Thus the transient stress is released in a manner that is totally transparent to the interface and applications of the transmitting station.

## Fairness Evens the Load

Fairness is another transparent mechanism that LANswitch uses to control the flow of traffic in the switch. LANswitch uses fairness to maintain balanced, non-proliferating queues of traffic in all the switch ports buffers, which eliminates pressure points in the switch. This is best illustrated in client/server configurations where many stations access a single server. With fairness imposed, each client port is given a regular turn to transmit ensuring that no single switch port will be able to monopolize the server. However, for some clients, equal access is not enough.

Let’s assume that at any given moment, one client is transmitting much more data than its peers, and even though its switch port is given regular (evenly divide between all transmitting clients) access to the server, it is unable to send all the data that the client station is generating. When this happens, leftover packets are stored in the client port buffer, waiting for the next turn to access the server. Meanwhile, the client continues to generate and transmit volumes of data, adding even more packets to its switch port buffer. Gradually, the switch port buffer fills up. At the same time, the port buffers of other clients remain relatively empty.

When uneven data loads cause uneven build-up in the client port buffers, LANswitch steps in and automatically corrects the imbalance by activating a special fairness algorithm. The switching fabric is clever enough to identify the most stressed port and give it precedence over any other port that is less loaded. This means that the more demanding station (which is most likely to fill its port buffer) will receive “preferred” access to the server. Most of the time, combining a "big pipe" server with optimized fairness algorithms for balancing input and output queues is sufficient to transparently diffuse typical overload conditions.

## Back Pressure eases rush-hour Gridlock

If these transparent internal mechanisms have been applied but both the source and destination ports remain flooded, then, and only then, is back pressure activated on the Ethernet telling all clients to cease transmission (so packets will not be lost). At the instant that back pressure is activated, any NIC residing on that segment will stop sending packets. The back pressure period is typically very short compared to the Ethernet time scale. In most cases, the port-to-switch transmission rate of 1.28 Gbps (i.e. the source port empties three 500 byte Ethernet packets from its buffer in less than 10 microseconds) will keep the back pressure period negligible.

When back pressure is applied due to a source port, all transmissions on that port’s LAN segment are temporarily suspended, even peer-to-peer transmissions that never travel through the switching fabric and do not affect the switch port buffers. Most of the time, the duration of this halt is negligible. However, extreme cases of prolonged heavy traffic on the network could extend the back pressure period, causing longer halts on the segment. At first glance, a segment-wide halt of this nature would appear to adversely affect LAN performance. But in truth, the back pressure mechanism actually enhances LAN performance by effectively increasing bandwidth and thereby, decreasing the time that stations must wait to transmit (i.e., segment “hold time”). To understand why this is so, we must take a closer look at the traffic patterns of switched LANs.

## Back Pressure and LAN Performance

To examine the effect of back pressure on LAN performance we must differentiate between two traffic patterns. One is traffic that travels through the switch, while the other is local traffic that remains on the local segment.

## Client/Server - Traffic Destined for the Switch

Today, most networks are client/server applications. By definition, almost all network traffic in a client/server application is between the clients and the server. Local traffic between clients is typically minimal. The rule of thumb in configuring a switched client/server LAN is to “position” the server on one side of the switching fabric (with a dedicated switch port), while the clients sit on the other side of the switch (generally sharing switch ports).

While this configuration optimizes efficiency by letting the switch handle the flurry of activity between clients and the server, it also makes switch port buffers susceptible to overflow.

For example, assume that a few clients share switch port A and one of them is currently in the midst of a lengthy transmission to the server. Combined with other factors, this traffic stream could overload port A’s buffer and at this point, if client stations continue to transmit, their packets will be lost. It is better to pause this traffic flow than risk losing it. Hence, back pressure is applied to the switch port causing all stations connected to port A to stop transmitting.

Has the performance of other stations on the client/server network been compromised? On the contrary.

	We already determined that on client/server networks, local traffic will be rare, so the brief back pressure period has virtually no measurable affect on the few peer-to-peer conversations that might occur.

	Furthermore, heavy traffic toward the switch dominated the segment, pre-empting any local traffic and causing the switch port buffer to overflow. Without back pressure, this traffic stream would - at best, cause a collision, and - at worst, cause packets to go astray.

	If, as often happens, client A and B attempt to transmit at the same time, a collision would occur and both packets would have to be retransmitted. Each station would back-off and then retry. Since the back pressure period is usually shorter than a normal Ethernet collision recovery period, it has less net effect than a normal Ethernet operation.

	What back pressure does is make sure that no packets are dropped by the client’s overloaded switch port. In the absence of such traffic control these waiting packets would not only go astray, but would significantly reduce the effective bandwidth by generating many more retries with continuous collisions.

As you can see, back pressure significantly improves network performance by preventing lost packets and wasteful retransmissions.

## Collapsed Backbone Segment Switching - Switch Traffic versus Local Traffic

We’ve determined that back pressure improves performance on client/server networks, but what about switched segment collapsed backbone networks, where an entire Ethernet segment shares a single switch port? In addition to switch traffic between segments, there is lots of local traffic on each segment.  What happens when switching buffers fill up and back pressure suspends transmission on a whole segment? To answer these questions, we need to take a closer look at the dynamics of Ethernet segment switching.

The rule of thumb in switched segment networks is to determine the size and number of Ethernet segments needed to accommodate performance requirements and then to position the switch in middle providing a high speed connection between all segments. Typically, the traffic on this type of network is a mix of switched packets between segments and local traffic within segments. Bear in mind that the local traffic never leaves the segment and therefore does not affect switch ports at all. Only heavy traffic between segments affects the switch port buffers and could cause back pressure to be activated.

Now, let’s assume that stations on segment A are sending lots of traffic through the switch, causing segment A’s port buffer to fill up and back pressure to be applied.

Has performance on the segment been compromised? On the contrary.

	First, we must remember that buffer overload on switched segments occurs much less frequently than on client/server ports because most of the traffic is local. Local transmissions never enter the switch, and therefore, do not contribute to the load on the switch port.

	Secondly, Ethernet’s stochastic nature limits the real bandwidth on shared segments much more than a few microseconds of back pressure. Switched segments typically have a greater number of stations per port. The larger the segment the smaller the effective bandwidth. As bandwidth is reduced, so is the fraction of packets that actually enters the switch port.

	Thirdly, in real terms, only a very small fraction of local conversations cannot take place while back pressure is active on the segment. Remember, traffic toward the switch dominated the segment putting local conversations on hold. If back pressure is not applied, this traffic stream will - at best, cause a collision, and - at worst, cause packets to be lost.

Flow control not only eliminates retransmissions, but also enables the switch to efficiently handle buffer overflow by providing a respite which is negligible in terms of Ethernet protocol, but sufficient enough to accelerate restoration to full bandwidth without retransmission overhead.

By preventing lost packets and the wasteful retransmissions back pressure actually increases available bandwidth and reduces “hold time” on the segment.

This papaer describes the LANswitch Ethernet and Fast Ethernet mechanism for Congestion management