# LSE-PM LANswitch Port Monitor
Installation Guide

## OVERVIEW

This installation guide is divided into three main sections:

- Description: A general description of the LSE-PM, including interfaces, LED indicators and a front panel illustration.

- Using the LSE-PM: Instructions for connecting the LSE-PM for local or remote packet analysis.

- Front Panel Components: Module diagram and explanations of all the diagnostic LEDs

- Installation: Step-by-step instructions for installing the LSE-PM in the hub.

## DESCRIPTION

The LSE-PM is a switch monitoring tool which complements Madge’s SMONMaster to provide a comprehensive switched LAN monitoring system.

Switched traffic is not broadcast to all ports; it is only transmitted to those ports to which traffic is directed. Therefore, you cannot monitor switched traffic by simply inserting a monitoring device in a switch port, as you can with shared traffic.

The LSE-PM copies packets from the Cellenium bus and sends them to an Ethernet port. A monitoring device such as a packet analyzer can then be connected to the Ethernet port and used to examine the packets.

The 1.28 Gbps Cellenium bus carries up to 128 full speed Ethernet conversations, while the LSE-PM transfers packets to a single 10 Mbps Ethernet port. Filters are therefore defined which allow selected packets to be captured.

The LSE-PM complements SMON which monitors the Cellenium bus and provides statistics about traffic patterns. SMONMaster can be used to discover the Top N hosts in a particular category (those that receive or transmit the highest number of broadcast  or error packets for example). The LSE-PM can then be used to pick out the packets transmitted by those hosts and send them to its Ethernet port for more detailed analysis.

## Using the LSE-PM

The LSE-PM can be used to provide packets for either local or remote monitoring and analysis. In order to perform local analysis, you will need a management station running MultiMan Console software and an analyzer, both connected to the hub containing the LSE-PM.

For remote monitoring and analysis, a shared Ethernet module and an RMON probe (e.g., Madge’s IERP) or remote packet analyzer are required in addition to MultiMan and RMONMaster™ on a management workstation.

Each of these options is described in detail below.

### Local Monitoring and Analysis

The LSE-PM Ethernet port can be connected directly to a packet analyzer. The ports or hosts to be monitored are selected from MultiMan, while the monitoring is done at the analyzer which is connected to the hub.

| Figure 1 – Output to a packet analyzer |  |
| --- | --- |

In order to connect the LSE-PM port directly to the analyzer, perform the following steps:

- Insert the LSE-PM into the MultiNet hub.

- Connect the analyzer to the LSE-PM.

- Select copied traffic using MultiMan.

### Remote Monitoring and Analysis

A remote probe is a device for monitoring network performance in real time and reporting in-depth network statistics over the network. Madge’s network monitoring solution includes the IERP – an Ethernet RMON probe module for MultiNet hubs, and RMONMaster – a MultiMan add-on application which analyses the data from the IERP, and displays RMON statistics and packet level analysis. RMON probes from other manufacturers can also be used, as can a remote packet analyzer.

Connecting the LSE-PM to the IERP allows both the selection of the ports to be monitored, and the monitoring of those ports to be done from the same management station (using MultiMan and RMONMaster).

| Figure 2 – Output to a combined monitoring and management station |  |
| --- | --- |

#### Connecting the LSE-PM to a Remote Packet Analyzer or RMON Probe

- Insert the LSE-PM.

- Find out the MAC address of the packet analyzer and enter it in the Monitor MAC Address box in the LSE-PM configuration window in MultiMan.

- Connect the packet analyzer to the LSE-PM.

- Select traffic to be copied by the LSE-PM using MultiMan.

#### Connecting the LSE-PM to an IERP

- Insert the LSE-PM.

- Insert a shared Ethernet module (e.g., the LE-140XTN) into the hub. The module must be connected to the Ethernet backplane on buses 1 to 3 only.

- Insert the IERP into the hub and connect it to the same Ethernet bus as the Ethernet module in step 2. Ensure the IERP interface is not connected to that bus.

- Connect the monitoring port on the LSE-PM to a port on the Ethernet module.

- Enter the MAC address of the IERP in the Monitor MAC Address in the LSE-PM’s configuration window in MultiMan.

- Select traffic to be copied by the LSE-PM using MultiMan.

| Note: Check the Virtual LAN! Ensure that the hosts or ports to be monitored are on the same Virtual LAN as the LSE-PM. If this is not the case, no packets will be captured from the hosts or ports of interest. An LSE-PM module which is connected to the Global Virtual LAN will capture traffic from all Virtual LANs apart from Generic. |  |
| --- | --- |

## FRONT PANEL COMPONENTS

Table 1: LSE-PM Front Panel Diagnostic LEDs

| LED Indicator | Description | State | Means... |
| --- | --- | --- | --- |
| TX | Transmission Activity | ON | Transmitting to the port |
| RX | Receive Activity | ON | Receiving from the port |
| C | Collision Status | ON | Collision detected |
| TF | Transmit Buffer Status | ON | Port transmit buffer is full (receive buffer from bus is full). |
| RF | Receive Buffer Status | ON | Port receive buffer is full (transmit buffer to bus is full). |
| Link | Port Status | ON 1 BLINK 2 BLINKS OFF | Port enabled and connected Link Test Fail (LTF) Partition Port disabled |

Figure 3: LSE-PM Front Panel

## INSTALLATION

### Module Installation

#### Inserting the LSE-PM in the MultiNet Hub

The LSE-PM occupies one slot in the MultiNet enclosure and can be inserted into any available position. The module may occupy LET-36 hub slots #1 to 15 (inclusive), LET-20 hub slots #1 to 9 (for slot #10 configuration please refer to the LET-20 installation guide), or LET-10 hub slots #1 to #4 (inclusive, numbered from the bottom).

The LSE-PM can be installed or removed while power is applied to the hub.

- Insert the module carefully into the hub according to the guidelines described above.

- Fasten the module by means of its two screws. Do not overtighten the screws.

- Connect a packet analyzer or IERP probe to the RJ connector as described in Page 2.

Catalogue number: IG-129020 Rev.0