|  | Application Note Virtual LAN Routing |
| --- | --- |

# 1.0	Overview

Workgroup computing needs are well met by the combination of LANNET’s LANswitch and virtual LANs. The LANswitch switched Ethernet environment provides the necessary dedicated bandwidth, while the virtual LAN facility allows logical network segmentation for  benefits such as firewalls for broadcast containment and security.

Routers may then be used to provide the desired connectivity between users and resources belonging to different virtual LANs.

This application note describes how to use LANNET’s integrated routers to provide this connectivity.

Thus, virtual LANs may be implemented today, using existing products and familiar, well-tested configurations. This allows LANNET customers to gain virtual LAN benefits today while meeting all connectivity requirements.

The method employed is that interfaces of the routers are connected to the various virtual LANs, whether physically or logically, and then the routers are used in the “traditional” manner, using the existing networking software and methodology to provide inter virtual LAN connectivity. The following drawing illustrates this approach using the IEFN (Integrated Ethernet Feeder Node) router.

# 2.0	Workgroup Computing Requirements

“Workgroup computing” and “Client/Server Computing” are buzzwords that are frequently heard. While they sound self-explanatory, in practice they mean different things to different people. For the LAN manager and LAN implementer, these business models, increasingly common, mean new requirements to be met by the LAN installation:

## Dedicated Resources

Workgroups typically require shared access to resources that are dedicated to workgroup needs, and serve no other group. Such resources may be the “servers” in “Client/Server computing”, video conferencing, shared access to a super-computing engine, file and application servers, etc.  These resources are usually dedicated to a certain purpose, and are not available for common access by all members of the organization.

## Change Management

An important characteristic of workgroup and client/server computing, and indeed of the modern work environment in general, is frequent change. People move location. Workgroups change membership frequently, adding and shedding members. New resources come and go, from fax and application servers, to supercomputers.

In the face of all of this chaos the LAN manager must cope, and provide the users with uninterrupted services. A user may move overnight to the new department, but still expects to be able to log-in, read e-mail, print, etc. the very first time he sits down at his new desk. A person is notified of being assigned to a new project - and 5 minutes later expects to have full access to the project's dedicated resources.

## Separation From The Rest Of The Organization

In order to keep the workgroup as an integral entity, and to provide it with the required resources and level of service, it must frequently be insulated and separated from the general workload of the enterprise.

In a similar vein, the other enterprise users frequently have to be protected from the new group. New groups starting out are a common cause of excessive resource consumption whether intentionally, or as start-up trials (and especially errors).

In designing a LAN solution for workgroups, we usually strive to give the workgroup a “private” LAN, so its workload does not affect that of other users and workgroups, and so that its work is not subject to disruptions and interference from outside the workgroup.

Security considerations also frequently mandate such separation.  Workgroup non-members must usually be restricted in their access to workgroup resources and information.

## Connectivity To The Rest Of The Organization

At the same time, workgroup members still remain part of the enterprise, and some level of connectivity must be maintained. Frequent needs are for access to common databases, electronic mail interchange with the rest of the enterprise and the outside world in general.

# 3.0	The LANswitch Solution

LANNET’s LANswitch provides an environment where each port has access to a high speed (1.28 Gbps) bus, over which Ethernet communication between the ports is carried out.

The main benefit of using LANswitch ports is that there is no contention between the ports over media access, and no collisions on the high-speed bus. Thus, each port perceives a dedicated Ethernet connection to any chosen LANswitch destination, and each port can utilize the full Ethernet bandwidth, with obvious performance gains.

In this manner LANswitch delivers the bandwidth necessary for modern client/server applications.

In the factory default configuration, where no virtual LANs have been configured, all LANswitch ports can communicate with any other port connected to the LANswitch environment.

While this may be a desired state, there are benefits to partitioning the network into multiple logical groups:

# 4.0	LANswitch Virtual LAN Requirements

- Broadcast Containment

- Broadcasts are delivered to all ports in the network, whether they need to receive them or not. If applications used generate a significant amount of broadcast traffic (e.g. UDP broadcasts are sometimes used as an information distribution method) segmentation of the network can be used to limit broadcast propagation to only the relevant ports.

- Security & Accountability

- Sometimes it is not desired to have uncontrolled access from every port to every port. Some examples of this are :

- If a port is unreachable except from authorized ports, its security cannot be breached no matter how clever the content of the packets generated at non-authorized ports.

- Limiting the ports receiving Novell IPX SAP broadcasts from a particular server, limits the ports to which that server is accessible for login.

- Access to costly resources (e.g. WAN lines, color printers) may be limited for both security (prevent unauthorized entries, prevent information leaks) and accountability reasons (limits on who can use a particular resource to a selected subset of ports).

- Firewall Construction

- Separating the network into logically separate LANs is a basic step in constructing firewalls to protect the network from misuse and mishaps, and to enable conformance to strict secrecy standards. For example, such a logical separation may be used by a service provider to allow proprietary information of clients to be kept apart.

- Matching Network Organization to Corporate Structure

- Modern corporations in many cases work in teams of various sizes and compositions.  LANswitch virtual LANs allow each team to have “its own” LAN, regardless of the physical location of its members.

- Keeping Existing Configurations

When upgrading existing network installation to a LANswitch environment, virtual LANs may be used as direct replacements for former physical LAN segments, thus allowing existing software configurations to be kept unchanged, greatly reducing the effort required in performing the upgrade.

# 5.0	LANswitch Virtual LANs

Grouping users and resources in separate groups, with dedicated bandwidth and easy changes is the natural mode of work for LANswitch ports partitioned into virtual LANs.

LANswitch virtual LAN connected users naturally get the desired degree of separation from other workgroups and the rest of the enterprise. The bandwidth allocated to each port is not affected by external parties changing their demand, so dedicated bandwidth allocation is achieved. Since ports can only communicate with ports assigned to the same virtual LAN, logical insulation, as well as security of workgroup data and access to workgroup resources are guaranteed.

Up to 255 different LANswitch virtual LANs may be defined, and in addition resources may be assigned to the “global” virtual LAN which is accessible to and from all other virtual LANs.

In this environment change is easy to cope with, as users may be moved into or out of a workgroup, and thus have their LAN access changed, at the click of a mouse.

So, to implement Workgroup computing, we may simply assign all members of a particular workgroup, and only members of that group, to a particular virtual LAN, and connect all of the workgroup's resources to LANswitch ports assigned to the same virtual LAN.  Simple, Easy, Efficient.

# A New Problem ?

However, a new problem seems to threaten an otherwise ideal picture. While the “separation” requirement is met, how do we cope with the “connectivity” requirement?

Virtual LAN users may only communicate with users in the same virtual LAN, and with users and/or resources who belong to the “Global” virtual LAN. This seems to leave the users with no way to communicate with users who belong to other workgroups, assigned to different virtual LANs.

So, while using LANswitch and virtual LANs answers most requirements, we need to find a way to provide the required connectivity between virtual LANs.

# Comparison With The “Traditional” Situation

This situation is not new. For many years now networks have been required to provide both separation (mainly for load distribution) and connectivity. The most common solution is to use some sort of partitioning at the network layer level to achieve the separation, and use routers to achieve controlled connectivity between the sub-groups.

As an example using the most common networking protocol, IP, network users are assigned to different “IP subnetworks”. Each user can directly communicate with any user or resource assigned to the same IP subnet, but must use the a router as an intermediary to communicate with users and access resources in other subnets.

Note that this has nothing to do with the physical arrangement of the network. Users may be connected to the same physical media, and still belong to different sub-networks. Two adjacent users, connected to the same Ethernet segment, may still need the router to serve as an intermediary to exchange messages.

Using a router where direct communications are possible may seem strange, but it affords the flexibility to partition users into logically separate workgroups, and use the same networking tools and protocols whether the destination belongs to the same physical LAN or to a remote one. In this way the user need not know where or how the destination user is connected.

This implementation is so common, that communication software, and most operating systems support it as a matter of course, with no user intervention. It is also a familiar configuration and there is a lot of accumulated experience with it to make it work as desired. This forms the basis of our proposed solution.

# 6.0	Routing In LANswitch Virtual LAN Environment

When LANswitch virtual LANs are used, a LANNET Router is connected physically to LANswitch ports, which are then logically assigned to the various virtual LANs used (or to the Global virtual LAN). Network layer protocols are then used to let the stations in the virtual LANs use the router for connectivity crossing virtual LAN boundaries.

LANNET integrated routers support up to four physical connections, with each one possibly supporting several logical sub-networks. (Depending on the networking protocol used, more than one logical network may be assigned to the same physical connection).  In this manner users in each virtual LAN see a router port available on their virtual LAN, and use that connection to send traffic destined to another virtual LAN.

# 7.0	Implementation Examples

The following sections describe various possibilities of achieving this connectivity, illustrating the point with IP, IPX and DECnet. For each case both a logical view and a physical drawing of the configuration is shown.

## 7.1 IP Routing Using A Router Port Connected To The Global Virtual LAN

One of the IEQR (Integrated Ethernet Quad Router) four Ethernet interfaces is connected to a Global port and has IP Network addresses X and Y.

This Cable connects the router with a LANswitch port Assigned to the GLOBAL Virtual LAN

Logical

View

Physical view,  showing the Actual modules and interconnections

Note:	The switch port which connects to the router MUST be in Global mode. Multiple IP subnets can be configured on a single router port, the limit being dependent on the available router memory, which in turn depends on the number of protocols and tasks.

## 7.2 IPX Routing

If a client from virtual LAN 1 wants to connect to the Novell server on virtual LAN 2, the packets have to be routed through the router. This example shows an IEFN (Integrated Ethernet Feeder Node) multi-protocol router, which has 2 Ethernet interfaces, and two WAN ports (X.25, Frame-Relay, etc.).

These cables link the LANswitch ports to the LE140XTQ, from which the data will be passed over Shared Ethernet buses (on the hub’s backplane)  to the router.

The LANswitch ports are each assigned to one of the two Virtual LANs

The LE140XTQ is required to provide the connectivity between the switched Ethernet environment and the internal shared Ethernet segments used by the IEFN router.

Notes :	In this configuration the IPX router and Novell servers must NOT be connected to Global ports. The reason is that the IPX protocol does not allow two different IPX networks on the same physical network.

## 7.3 IPX Routing Using Two NICs In The Novell Server

Sometimes it is necessary, usually for performance reasons, to have two Network Interface Cards (NICs) in the server. When this is done, the server automatically performs IPX routing between the two networks - this is done by a software task on the server CPU. This is not only slow in itself, but also greatly degrades the server’s general performance due to its consumption of CPU resources.

The configuration illustrated below shows how to create the desired configuration, while having all the routing load carried by the external router. Internal routing should be turned off in the server, so all routing is done by the external router, with no server performance load. This is done by using an NLM (Novell Loadable Module) such as the “Route-Off “ NLM software sold by the NSI.

For additional performance gain, the Netware LIPX.NLM (Long IPX module) may be run on the NOVELL servers (assuming Netware version 3.1X and later)

Clients in this network can attach to the NOVELL-1 server through the router

Clients in this network can directly attach to both servers (without router intervention)

## 7.4 Routing In A Mixed Protocol Environment

Using the same reasoning, we can easily accommodate a multi-protocol environment, where users in the two virtual LANs get router-controlled access to resources in the other group, and to the Wide Area Network.

Note that Each port can be individually assigned to a Virtual LAN.

The separation here to two modules is just for graphical clarity

## 7.5 Using The IEQR Router For Mixed Protocol Routing

Using the LANNET Integrated Routers, up to four virtual LANs may be served. The IEQR (Integrated Ethernet Quad Router) has four Ethernet interfaces, and each one can be separately attached to a virtual LAN as illustrated below.

Note also that some of the servers are connected to the High Speed (1.28 Gbps) bus using 100BaseT (100 Mbps) connections.

The following drawing illustrates the physical implementation of this application. While seemingly complex, this is actually a straightforward extension of the previous examples.

The workstations are groups into four virtual LANs, implemented by assigning the LANswitch ports to which the stations are connected to the desired virtual LANs. Workstations in each virtual LAN  need not be using the same protocol - in the illustration each virtual LAN carries DECnet, IP and IPX.

A patch cable is used to connect one of these virtual LANs to the router’s front-panel interface.  The other three virtual LANs are connected by patch cables to the LE140XTQ ports, providing connectivity to the shared Ethernet segments in the hub’s backplane. One such connection being used for each virtual LAN.

The IEQR router receives data from the four Ethernet segments and forwards it to the desired destination.

Three of the IEQR’s four Ethernet Interfaces are always connected to the hub’s backplane, and through the LE-140XTQ to the LANswitch ports

Note that the GLOBAL virtual LAN may NOT be used for IPX and therefore for connection to the router. This results from the aforementioned IPX limitation of one network per LAN.

## 7.5 Using The IEFR For Mixed Protocol Routing With An FDDI Backbone

By substituting an IEFR (Integrated Ethernet to FDDI Router) router instead of the IEQR in the previous example, we not only provide multi-protocol routing between four virtual LANs, but also provide all users with access to an FDDI backbone. (The IEFR provides four Ethernet interfaces, and one FDDI connection, Fiber or Twisted Pair)